Lab 5

Vision-Guided Pick-and-Place with Colored Blocks

Accompanying MATLAB file(s):  UR_VisionPick_Lab.m, capture_realsense.m, detect_block.m, fit_pixel_to_base.m, pixel_to_base.m, test_vision_offline.m

Author: Ryan Cullen, SFSU MSME 2026

Overview

In this lab you close the loop between seeing and moving. A wrist-mounted camera photographs the workspace, MATLAB finds a colored block in the image, a calibration you build converts the block’s pixel position into robot base coordinates, and the robot picks the block up wherever it lies. This answers the question Lab 4 left open: how a robot can pick objects whose position is different every time. The motion side reuses what you built in Labs 2 and 4; the new material is the camera, the color detection, and the pixel-to-robot calibration.

Prerequisites

•  Labs 1 through 4 completed, with the robot networked to your PC (Lab 2)

•  Intel RealSense D435if mounted on the wrist (mount CAD is in 1. UR Docs / UR Low-Cost Camera Setup) and connected to a USB 3.0 port

•  Pendant payload updated for the camera plus gripper (Lab 1, Section 3.2)

•  The guide "RealSense Set Up With MATLAB" [1] completed on your PC

•  MATLAB Image Processing Toolbox

•  Two or three colored blocks of one color, on a flat table that does not move

Learning Objectives

•  Bring up a RealSense camera in MATLAB and capture RGB images from a program.

•  Segment a colored object with HSV thresholding and locate its centroid in pixels.

•  Calibrate an affine mapping from image pixels to robot base coordinates using TCP-touched reference points, and judge its quality from the residuals.

•  Command Cartesian motion to a computed target with sendCartesianPoseAndWait.

•  Run a vision-guided pick and explain the conditions under which the calibration stays valid.

References

[1]  RealSense Set Up With MATLAB (Box: 1. UR Docs / UR Low-Cost Camera Setup / UR Camera Setup / MATLAB Camera Standard Operating Procedure)

[2]  Lab 4 manual and UR_PickPlace_Lab.m (waypoints, gripper, motion calls)

[3]  MathWorks Robotics System Toolbox Support Package for UR Series Manipulators, urRTDEClient reference

Background: How the Pieces Fit

Three ideas make this lab work. First, the robot always takes the picture from the same IMAGING POSE, so the camera sees the table from one fixed viewpoint. Second, the blocks always sit on the same table plane. Together these mean each image pixel (u, v) corresponds to one point (X, Y) on the table, and that correspondence is well approximated by an affine map:

[X ; Y] = A * [u ; v ; 1]        (A is a 2 x 3 matrix you calibrate in Part 4)

Third, the pick height Z and the gripper orientation never need to be computed: you teach them once by recording a reference pick pose on a real block, and the script replaces only X and Y with the detected position. The key functions are summarized below.

| Function | Description |
| --- | --- |
| img = capture_realsense() | Grab one RGB image from the RealSense camera. |
| [uv, mask] = detect_block(img, hsvLow, hsvHigh, minAreaPx) | HSV threshold; returns the largest blob’s centroid in pixels, and the mask for tuning. |
| [A, res, rms] = fit_pixel_to_base(UV, XY) | Least-squares fit of the affine map from your calibration points, with per-point residuals in mm. |
| XY = pixel_to_base(A, uv) | Apply the map to one detected centroid. |
| sendCartesianPoseAndWait(ur, pose) | Move the TCP to pose = [thetaz thetay thetax x y z] (radians and meters, base frame). |

Table 1 - Key functions used in this lab.

UR_VisionPick_Lab.m is staged: it runs as far as your entered values allow and then stops safely, so you run the same script at every part of this lab. All camera code lives in capture_realsense.m; you never call the RealSense SDK directly.

Part 1 - Camera Setup

Task 1: Mount and verify the camera.

•  Mount the camera on the wrist bracket and route the USB cable so no motion in your workspace can snag it.

•  On the pendant, update the payload mass and center of gravity for gripper plus camera (Lab 1, Section 3.2). A camera is light but it is cantilevered; skipping this causes drift in FreeDrive and sloppy stops.

•  Complete the guide [1] on your PC if you have not already: SDK, firmware, and the MATLAB wrapper.

•  Verify in MATLAB:

img = capture_realsense();  imshow(img)

You should see a live photo from the wrist camera. If MATLAB reports that the wrapper is missing, the error message tells you what to fix; the full procedure is in [1].

Part 2 - The Imaging Pose

The calibration you build in Part 4 is tied to one viewpoint. If the camera views the table from anywhere else, the calibration is wrong. So the first waypoint of every vision pick is a fixed IMAGING POSE, and you calibrate from that exact pose.

Task 2: Choose and record the imaging pose.

•  Using FreeDrive (Lab 1, Section 7) or Move > Joints, position the arm so the camera looks straight down at the workspace from roughly 0.4 m above the table.

•  Check the view with capture_realsense and imshow: the whole area where blocks may lie must be visible, and the gripper fingers should not block the view.

•  Record the six joint angles from the pendant into Table 2, and enter them into TASK 2 of UR_VisionPick_Lab.m.

•  Run the script. It should move to the imaging pose and display the camera view, then stop.

| J1 | J2 | J3 | J4 | J5 | J6 |
| --- | --- | --- | --- | --- | --- |

Table 2 - Imaging pose joint angles (degrees).

Part 3 - Detecting the Block

A color in RGB mixes brightness into all three channels, which makes thresholds fragile. HSV separates what the color is (hue) from how strong it is (saturation) and how bright it is (value), so a colored block can be isolated with simple ranges on each channel. All three channels run from 0 to 1 in MATLAB. One special case: red sits at the wrap-around of the hue circle, so a red band uses a LOW bound above the HIGH bound (for example low h = 0.95, high h = 0.05) and detect_block handles the wrap for you.

Task 3: Tune the thresholds.

•  Place one block near the middle of the workspace.

•  Run the script. Figure 2 shows the mask overlaid on the image, plus the detected centroid.

•  Adjust hsvLow and hsvHigh in TASK 3 until exactly one clean blob covers the block: start by narrowing hue, then raise the saturation lower bound to reject the gray table, then adjust value only if shadows or glare interfere.

•  Record your working thresholds in Table 3. If the room lighting changes later, expect to re-tune.

| Block color | hsvLow [h s v] | hsvHigh [h s v] | Centroid (u, v) px |
| --- | --- | --- | --- |

Table 3 - Working HSV thresholds.

Part 4 - Pixel-to-Base Calibration

Now you teach the robot where pixels are in its own coordinates. You will place the block at four known spots, and for each one record BOTH what the camera sees (the centroid in pixels) AND where the robot says the block is (the TCP position when touching it). fit_pixel_to_base then finds the 2 x 3 matrix A that best maps one to the other, and reports the leftover error at each point (the residuals).

⚠️ The pendant and readCartesianPose report the position of the ACTIVE TCP. Make sure the TCP configured in Lab 1, Section 3.2 matches the physical tool point you touch the block with, or every calibration point inherits the same offset.

Task 4: Calibrate. For each of four placements, spread wide so they roughly frame the working area:

•  Place the block, run the script, and record the detected centroid (u, v) in Table 4.

•  Without moving the block, jog the TCP (Move > Tool, or careful FreeDrive) until the tool point touches the center of the block’s top face.

•  Read X and Y from the pendant Move screen or with readCartesianPose(ur) in MATLAB (position is elements 4 to 6), and record them in Table 4. Then retract straight up.

•  Enter all four rows into UV and XY in TASK 4 of the script and run it. Record the residuals and the RMS.

•  Under 3 mm RMS is excellent; 3 to 5 mm is acceptable for 25 mm blocks; above 5 mm, redo the placement with the worst residual. A single bad row usually means its UV and XY came from different placements.

| Point | u (px) | v (px) | X (m) | Y (m) | Residual (mm) |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| RMS (mm): |  |  |  |  |  |

Table 4 - Calibration points and fit quality.

Part 5 - Vision-Guided Pick

Task 5: Teach the two remaining poses.

•  Reference pick pose: place a block anywhere, jog the gripper into a perfect grasp position around it (fingers open, block centered, gripper vertical), then run readCartesianPose(ur) and paste the six numbers into ref_pick_pose in TASK 5. This pose supplies the orientation and the pick height for every future pick; the script replaces only X and Y.

•  Place waypoint: jog to where blocks should be dropped off and record the joint angles into place_pos.

Task 6: Run it.

⚠️ Set the pendant speed slider to 10% for the first run. The robot will move to a position COMPUTED FROM THE IMAGE; a bad calibration sends it somewhere you did not teach. Keep the e-stop in hand. The script asks for confirmation before any pick motion.

•  Place the block somewhere new inside the calibrated area and run the script end to end: image, detect, convert, confirm, pick, place.

•  Repeat for five placements, moving the block each time. For each trial estimate how far from the block center the gripper closed (eyeball against a ruler is fine) and log Table 5.

•  If a pick misses consistently in one direction, revisit the TCP warning in Part 4 before touching the thresholds.

| Trial | Detected (Y/N) | Pick success (Y/N) | Grasp offset (mm, est.) | Notes |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

Table 5 - Vision-guided pick trials.

Part 6 - Going Further (Optional)

The D435if also measures depth for every pixel. The wrapper exposes it through the depth frame (get_depth_frame, then get_distance at a pixel). Try reading the depth at your detected centroid from the imaging pose and compare it with what you would predict from the camera height. Could you use this to pick blocks of two different heights with one calibration? What replaces the fixed pick Z?

Assessment Questions

Q1.  The calibration is valid only from the fixed imaging pose, for blocks on the table plane. Explain why, and describe what happens to the predicted X and Y if someone lowers the table by 20 mm without recalibrating.

Q2.  How did you choose your HSV thresholds, and what happened (or would happen) to detection when the room lighting changes? Name one change to the setup, hardware or software, that would make detection more robust.

Q3.  Compare your calibration RMS with the grasp offsets you observed in Table 5. List three distinct error sources between the camera image and the gripper fingers, and say which one you believe dominates in your setup.

Q4.  Your system picks round or square blocks because one taught orientation fits them all. What additional quantity would you need to pick long rectangular blocks lying at arbitrary angles, and how could you compute it from the detection mask?

Q5.  In Lab 4 Q5 you argued which programming approach suits picking from changing locations. You have now built it. What would still break if the blocks arrived on a moving conveyor, and what two additions would the system need?

Deliverable: completed Tables 2 through 5, and a vision-guided pick demonstrated live to your instructor.