## RealSense Camera: Setting up the Intel D435if with MATLAB

This document will outline the required steps to set up the Intel RealSense D435if Camera with MATLAB. The required items you need are listed below:

- Intel RealSense D435if

- Windows 10 or 11 PC

- USB 3.0 Type-A Port or USB-C Port (or compatible with adapter)

- USB 3.0 cable (the cable included with the D435i or an equivalent SuperSpeed cable)

- MATLAB (R2025b used for this guide; newer versions should work)

- Intel RealSense SDK 2.0 (V2.58.2 used for this guide; newer versions should work)

- Intel RealSense Viewer (installed with the updated SDK)

- MATLAB wrapper (installed with the outdated RealSense SDK)

## 1. Obtaining The MATLAB Wrapper

MATLAB cannot directly communicate with the RealSense camera’s SDK. We must use a wrapper to translate MATLAB commands to the RealSense’s C++ library to give MATLAB access to the camera’s data through functions such as realsense.pipeline(). The wrapper must be downloaded in an earlier release of the SDK, version 2.51.1, because it has NOT been included in the SDK since V2.56.2.

- Go to Intel RealSense SDK 2.0 v2.54.1 and scroll down to ‘Assets’ → click to download ‘Intel.RealSense.SDK-WIN10-2.54.1.5216.exe’

- Open the ‘Intel.RealSense.SDK-WIN10-2.54.2.xxxx.exe’ file and procced with the installer

- On the ‘Select Components’ page of the installer, ensure that ‘MATLAB Developer Package’ IS SELECTED → reference Fig. 1

- Proceed with the remaining steps of installer

Fig. 1 – MATLAB Selected in the SDK Installer

## 2. Camera Start-Up & Firmware

Before operating the camera in MATLAB, we must install the most recent SDK on top of the MATLAB wrapper to ensure that the camera has its firmware up to date. This step also ensures that the camera is fully operational.

- Go to Intel RealSense SDK 2.0 and scroll down to ‘Assets’ → click to download ‘RealSense.SDK-WIN10-x.xx.x.10647.exe’ (This guide uses RealSense.SDK-WIN10-2.58.2.10647.exe; newer versions should work)

Fig. 2 – RealSense Viewer

- Open RealSense Viewer (installed from downloading the SDK) → reference Fig.2 as to what should the program to appear as

- Plug RealSense D435if into PC’s USB 3.0 port (If using USB-C to USB-C cable, use USB-C port)

Fig. 3 – RealSense Viewer with D435if Connected

- Toggle the Stereo, RGB, and Motion modules on the left by clicking the red boxes

- Windows displaying the RGB, Depth, Gyro, and Accel data should appear → reference Fig.3 as what should the program to appear as

If the RealSense Viewer does not recognize your camera when you plug it into your PC, labeling it as ‘Intel RealSense D435’ (and/or other incorrect names), you must update the camera’s firmware. To do so:

- Go to Intel RealSense SDK 2.0 and scroll to the ‘Supported Cameras’ section → in the comments tab, open the ‘Firmware releases D400’ page → scroll to the’D400 Series firmware downloads’ section and download the MOST recent errata for the D435if

- In the RealSense Viewer, click the three lines labeled ‘more’ next to your camera on the left side of the screen

- Click on the ‘Update Firmware’ tab → reference Fig. 4 for support → find the ‘.bin’ file you downloaded from the ‘Firmware releases D400’ page and upload it to the camera

- The RealSense Viewer should now appear as Fig. 3

Fig. 4 – ‘More’ tab in the RealSense Viewer

The D435if camera should now display data when you toggle the modules on. If there are still issues with starting up the camera, reference Firmware Updates, RealSense SDK Updates, and general notices on the RealSense Website.

## 3. Operating The Camera In MATLAB

Now that we have installed the MATLAB Developer Package in the earlier version of the SDK and the up-to-date firmware in the most recent version of the SDK and ensured that the camera is operational in the RealSense Viewer, we are ready to control the camera in MATLAB. This manual is using MATLAB R2025b, however newer versions should work.

- Open MATLAB and navigate to the MATLAB folder within the SDK → C:\Program Files (x86)\Intel RealSense SDK 2.0\matlab

- You should see a ‘+realsense’ folder on the left → reference Fig. 5

Fig. 5 – MATLAB Folder Opened in MATLAB

- To check that MATLAB sees the wrapper, copy, paste, and run:

- which realsense.pipeline

in the command window → you should see a path ending in ‘\matlab\+realsense\pipeline.m’ → if not, troubleshooting may involve reinstalling the wrapper and/or ensuring MATLAB is in the correct folder

- To check that MATLAB sees the camera, copy, paste, and in the same prompt run:

- ctx = realsense.context();

- devices = ctx.query_devices();

- disp(devices)

In the ‘Workspace’ tab on the left side of the screen, you should see ‘device’ appear → reference Fig. 6

- Now you can run commands such as

- realsense.depth_example

- realsense.capture_example

To start up the camera in MATLAB and view its RGB, Depth, IMU, and Motion outputs → reference Fig. 7 & Fig. 8

Fig. 6 – ‘Workspace’ Tab in MATLABFig. 7 – Running ‘realsense.depth_example’ in MATLAB

Fig. 7 – Running ‘realsense.capture_example’ in MATLAB

Note: The ‘realsense.capture_example’ contains acceleration and gyroscope data. While not shown in Fig. 7, the acceleration data has units of meters per seconds squared (m/s²) and the gyro data has units of radians per seconds (rad/s). Further, the ‘r’ provided is the magnitude     (r = √ (x² + y² + z²) of the 3D vector created by both data sets, respectively.

## 4. References

- Updated Intel RealSense SDK 2.0

- Intel RealSense SDK 2.0 v2.54.1

- Firmware Updates

- RealSense Website

Author: Leo Nhuch, SFSU CARE Lab 2026